LCD Display GUI Design

Greenhouse Ventilation Controller  —  16×2 Character LCD + 4×4 Keypad

| Field | Value |
| --- | --- |
| Document | LCD Display GUI Design |
| Project | Greenhouse Ventilation Controller |
| Display | Waveshare LCD1602 I2C — 16 characters × 2 rows |
| Input | 4×4 membrane matrix keypad |
| Version | 0.1 (draft) |
| Date | 2026-03-30 |

# 1. Keypad Layout and Key Mapping

The 4×4 membrane keypad provides all navigation and data-entry input. Column D and the ‘*’ and ‘#’ keys carry fixed functions in every screen context. The numeric keys 0–9 are used exclusively for digit entry when the display is in edit mode.

| 1 | 2 | 3 | A ▲ UP |
| --- | --- | --- | --- |
| 4 | 5 | 6 | B ▼ DOWN |
| 7 | 8 | 9 | C ⏎ ENTER |
| * CLR | 0 | # OK | D ← BACK |

| Key | Label | Function in menus | Function in edit mode |
| --- | --- | --- | --- |
| A | ▲ UP | Move cursor up / previous item | Increment digit / previous option |
| B | ▼ DOWN | Move cursor down / next item | Decrement digit / next option |
| C | ⏎ ENTER | Confirm selection / enter submenu | Accept edited value and return |
| D | ← BACK | Go back one level / cancel | Cancel edit without saving |
| * | CLR | Return to idle screen from any depth | Backspace / clear last digit |
| # | OK | Shortcut: confirm current item | Confirm value (same as C) |
| 0–9 | Digits | Quick-jump to menu item by number | Enter digit at cursor position |

# 2. Access Levels

Three access levels control which screens and actions are available. The active session is shown in the rotating idle views and times out after a configurable inactivity period (default 5 min). Login requires a PIN; the PIN is validated against a bcrypt hash stored in NVS flash.

| Level | Login required | Screens accessible | Session timeout |
| --- | --- | --- | --- |
| Public | No | All idle / rotating views, alarm views, About, Main Menu (read-only) | — |
| Farmer | Yes — Farmer PIN | All Public screens + Manual Control, all Settings except Network & Access Control | 5 min (configurable) |
| Administrator | Yes — Admin PIN | All Farmer screens + Network Settings, Access Control (change PINs) | 5 min (configurable) |

# 3. Idle Behaviour and Screen Rotation

When no key is pressed for 5 seconds the display enters the rotating idle loop. Six information screens (S-01 to S-06) cycle automatically every 4 seconds. Pressing any key exits the idle loop and opens the Main Menu. S-06 (Session Info) is only shown when a Farmer or Administrator session is active.

An active alarm always overrides the idle loop: the alarm screen replaces S-01 and the ► indicator blinks at 2 Hz until the alarm clears. When multiple alarms are active they rotate every 2 seconds.

| Screen | Content | Cycle position | Condition |
| --- | --- | --- | --- |
| S-01 | Temperature & relative humidity + mode + clock | 1st (always) | Always shown |
| S-02 | Wind speed, direction, SAFE / ALARM indicator | 2nd (always) | Always shown |
| S-03 | State of all three windows (M1, M2, M3) | 3rd (always) | Always shown |
| S-04 | Operating mode and current date | 4th (always) | Always shown |
| S-05 | WiFi status and IP address | 5th (always) | Always shown |
| S-06 | Logged-in user and session time remaining | 6th | Only when logged in |

# 4. Navigation Map

The diagram below shows all top-level navigation paths. Any key from the idle loop opens the Main Menu. D (BACK) always returns to the parent screen; * returns directly to the idle loop.

IDLE ROTATION  S-01 → S-02 → S-03 → S-04 → S-05 [→ S-06]  ↺ loop

any key ↓

MM-01  MAIN MENU

├── Status             →  back to idle loop

├── Manual Control      →  [login if needed]  →  MC-01

├── Settings            →  [login if needed]  →  ST-01

├── Login / Logout      →  LG-01

└── About               →  AB-01

LG-01  Login: select role  →  LG-02  Enter PIN

├── correct PIN  →  LG-03  Welcome  →  return to caller

└── wrong PIN    →  LG-04  Error  →  retry (max 3) or D to exit

LG-05  Logout confirm  →  C=Yes: session ended  |  D=No: stay logged in

MC-01  Window list  →  MC-02  Window command screen

├── C on [OPEN]   →  MC-03  Moving … (relay active, countdown)

├── C on [CLOSE]  →  MC-03  Moving …

└── blocked by wind override  →  MC-04  Blocked message

ST-01  Settings menu

├── Climate       →  CL-01  [Farmer+]  → item → VE  edit value

├── Wind Safety   →  WS-01  [Farmer+]  → item → VE  edit value

├── Windows       →  WN-01  [Farmer+]  → item → VE  edit value

├── Time & Date   →  TD-01  [Farmer+]  → TD-02 / TD-03

├── Network       →  NW-01  [Admin]    → item → VE  edit value

└── Access Ctrl   →  AC-01  [Admin]    → AC-02 / AC-03 / AC-04

VE  Value edit  →  # or C to save  |  D to cancel

Alarm overlays A-01…A-04  appear on top of idle; any key → main menu

# 5. Idle Rotating Views

These six screens cycle automatically. Any key press interrupts the cycle and opens the Main Menu. The ► indicator at the right of row 2 on S-01 and S-02 blinks when a relevant alarm is active.

| T:+24.5C RH: 68 AUTO 22:34 S-01 Climate & Mode Any key → Main Menu (* = directly back here from anywhere) | Wind:12.3m/s Dir: NNE SAFE S-02 Wind Blinks ► when wind alarm active |
| --- | --- |

| M1:OPEN M2:OPEN M3:CLOSED S-03 Window States M1=Roof L, M2=Roof R, M3=N.Wall | Mode:AUTOMATIC Mo 30 Mar 2026 S-04 Mode & Date Mode: AUTOMATIC / STANDBY |
| --- | --- |

| WiFi: Connected 192.168.001.042 S-05 Network Status Shows AP mode IP when in access-point mode | Farmer: active Sess: 04:32 left S-06 Session Info [Farmer / Admin] Only shown when Farmer or Admin session is active |
| --- | --- |

# 6. Alarm Views

Alarm screens replace the idle rotation when an alarm is active. The first row blinks at 2 Hz (implemented by toggling the ‘!’ delimiters). Multiple simultaneous alarms rotate every 2 s. Any key press goes to the Main Menu (the alarm indicator remains visible in S-01 or S-02 of the idle loop until the condition clears).

| !WIND OVERRIDE! A-01 Wind Override All windows CLO. Any key → Main Menu. Alarm clears automatically when wind drops. ⚠ Overrides all Climate Control commands; windows cannot be opened manually. | !SENSOR FAULT! A-02 Wind Sensor Fault Wind: no resp. Any key → Main Menu. Auto-clears on next successful Modbus poll. ⚠ System enters fail-safe: treats wind as unsafe, keeps windows closed. |
| --- | --- |

| !SENSOR FAULT! A-03 T/RH Sensor Fault T/RH: no resp. Any key → Main Menu. Climate Control suspended until sensor recovers. | !MAN.OVERRIDE! A-04 Manual Override RRK-3: detected Any key → Main Menu. Clears when RRK-3 relay de-activates. ⚠ Climate Control suspended. Manual window commands still accepted. |
| --- | --- |

# 7. Main Menu

The Main Menu shows two items at a time. The ‘>’ cursor marks the selected item. A ▼ indicator in row 2 position 15 shows that more items are below. Items that require login are marked with ►; selecting them triggers the Login flow if no sufficient session is active.

MM-01  Main Menu — scrolling list (5 items, 2 visible)

| >Status Manual Ctrl ▼ MM-01a Top of list A/B = scroll C = select D = back to idle * = idle | >Manual Ctrl ► Settings ▲▼ MM-01b Scrolled down ► = login required for this item |
| --- | --- |

| >Settings ► Login ▲ MM-01c Further down Login item changes to "Logout" when a session is active | >About ▲ MM-01d Bottom of list ▲ = no more items below; B wraps to top |
| --- | --- |

# 8. Login and Logout

Login is triggered either from the Login menu item or automatically when the user selects a restricted action (Manual Control, Settings). The PIN is entered digit-by-digit; each confirmed digit is replaced by ‘*’. After 3 consecutive failures the login is locked for 60 seconds.

| Login as: >Farmer Admin LG-01 Select Role A/B = toggle role C = select D = cancel | Farmer PIN: >_ _ _ _ _ _ LG-02 Enter PIN (Farmer) 0–9 = enter digit * = backspace C/# = confirm D = cancel |
| --- | --- |

| Farmer PIN: >* * * _ LG-03 PIN entry in progress Entered digits shown as *; cursor advances automatically | Welcome, Farmer! Press C to cont. LG-04 Login success C = continue to requested screen D = return to main menu |
| --- | --- |

| Wrong PIN! 2 attempts left LG-05 Login failed C = try again D = back to role select ⚠ After 3 failures: "Locked 60s" message; login disabled for 60 s. | Logout Farmer? C=Yes D=No LG-06 Logout confirm C = end session and return to Public D = keep session active |
| --- | --- |

# 9. Manual Control

Manual Control is available to Farmer and Administrator. The window list shows current state for each window. Selecting a window opens its command screen where OPEN and CLOSE can be issued. A wind override blocks OPEN commands and shows a blocked message. The relay travel timer is displayed as a countdown.

| >M1 Roof OPEN M2 Roof OPEN MC-01 Window list [Farmer / Admin] A/B = scroll (M3 below) C = open command screen D = back | M2 Roof OPEN >M3 N.Wall CLSD MC-01b Window list (M3 visible) [Farmer / Admin] A/B = scroll C = open command screen D = back |
| --- | --- |

| M1: Roof Vent >[OPEN] [CLOSE] MC-02 Window command screen [Farmer / Admin] A/B = toggle between OPEN and CLOSE C = execute D = back | M1: Opening... Time left: 021s MC-03 Window moving D = abort movement (window stops in intermediate position) ⚠ Countdown from motor travel time (M1/M2: 21 s, M3: 171 s). |
| --- | --- |

| M1: BLOCKED! Wind override MC-04 Command blocked D = back to command screen * = idle ⚠ Shown when OPEN is attempted during active wind override. | M1: Dwell wait Retry in: 015s MC-05 Dwell timer active D = back; command auto-retries after dwell expires ⚠ Shown when a reversal command arrives before dwell timer expires. |
| --- | --- |

# 10. Settings

Settings are organised in six sub-menus. Farmer can access Climate, Wind Safety, Windows, and Time & Date. Administrator additionally has access to Network and Access Control. Changed values are persisted to NVS flash immediately on confirmation.

ST-01  Settings Menu

| >Climate Wind Safety ▼ ST-01a Top of settings (Farmer view) [Farmer / Admin] A/B = scroll C = enter D = back * = idle | >Network ► Access Ctrl ►▲ ST-01b Bottom of settings (Admin view) [Admin] ► = Admin only items; shown greyed if Farmer logged in |
| --- | --- |

CL  Climate Settings  [Farmer+]

Five configurable parameters. Selecting any item opens the Value Edit screen (VE).

| >Temp SP:+22.0C RH SP: 70%▼ CL-01a Climate menu (top) [Farmer / Admin] A/B = scroll C = edit D = back | >Hyst T: 1.0C Poll: 60s▲ CL-01b Climate menu (bottom) A/B = scroll C = edit D = back |
| --- | --- |

WS  Wind Safety Settings  [Farmer+]

| >Max: 12.0 m/s ExclSt: NNE▼ WS-01a Wind Safety menu (top) [Farmer / Admin] A/B = scroll C = edit D = back | >ExclEnd: NNW Hold-off: 300s▲ WS-01b Wind Safety menu (bottom) Excl Zone: compass direction of start/end of exclusion arc |
| --- | --- |

WN  Window Settings  [Farmer+]

| >M1 Dwell: 030s M2 Dwell: 030s▼ WN-01a Window menu (top) [Farmer / Admin] A/B = scroll C = edit D = back | >M3 Dwell: 060s M3 Travel:171s▲ WN-01b Window menu (bottom) Travel time = relay energised duration for full OPEN/CLOSE |
| --- | --- |

TD  Time & Date Settings  [Farmer+]

| >Set Time Set Date ▼ TD-01 Time & Date menu [Farmer / Admin] A/B = scroll C = enter D = back | Set Time: HH:[22] MM:[34] TD-02 Set Time A/B = +/− value C/# = next field D = cancel * = clear field ⚠ Fields cycle: Hours → Minutes → Confirm. Writes to DS3231 on confirm. |
| --- | --- |

| Set Date: [30]/[03]/[2026] TD-03 Set Date A/B = +/− value C/# = next field D = cancel ⚠ Fields cycle: Day → Month → Year → Confirm. Validates days-per-month. | Time & Date OK! Press D to exit TD-04 Time/Date saved D = back to menu * = idle |
| --- | --- |

NW  Network Settings  [Admin only]

| >WiFi: Client SSID: GH-Ctrl▼ NW-01a Network menu (top) [Admin] A/B = scroll C = edit D = back | >Password: **** MQTT: Enabled▲▼ NW-01b Network menu (middle) Password shown as *; edit opens character-by-character input |
| --- | --- |

| >MQTT Broker: 192.168.001.10▲ NW-01c Network menu (bottom) [Admin] IP entered digit-by-digit, dotted-decimal format | WiFi Mode: >AP Client NW-02 WiFi Mode toggle A/B = toggle C = confirm D = cancel ⚠ Change takes effect after reboot; confirmation screen shown. |
| --- | --- |

AC  Access Control  [Admin only]

| >Change F. PIN Change A. PIN▼ AC-01 Access Control menu [Admin] A/B = scroll C = select D = back | New Farmer PIN: >_ _ _ _ _ _ AC-02 Enter new PIN 0–9 = digit * = backspace C/# = confirm D = cancel ⚠ PIN must be 4–8 digits. Short PINs rejected with error message. |
| --- | --- |

| Confirm PIN: >_ _ _ _ _ _ AC-03 Confirm new PIN Re-enter same PIN to confirm. Mismatch → error, restart entry. ⚠ If confirmed PIN matches, hash is written to NVS immediately. | PIN changed OK! Press D to exit AC-04 PIN saved D = back to Access menu * = idle |
| --- | --- |

# 11. Value Edit

A generic edit screen is used for all numeric and option settings accessed from the Climate, Wind, Window, and Network menus. The current value is shown in row 1; the edit field is in row 2. The cursor ‘_’ shows the active digit position. The type of edit depends on the parameter.

| Temp Setpoint Edit:[+22.5_]#OK VE-01 Numeric edit (float) 0–9 = digit A/B = +/− 0.1 * = backspace C/# = save D = cancel | M1 Dwell (s) Edit:[ 030_] #OK VE-02 Numeric edit (integer) 0–9 = digit A/B = +/− 1 * = backspace C/# = save D = cancel |
| --- | --- |

| Wind Excl Start Choice:> NNE VE-03 Option / enum select A/B = cycle through options (N NNE NE ENE E … 360°) C/# = save D = cancel | SSID: [GH-Ctrl_ ]# VE-04 Text edit (SSID / hostname) A/B = cycle character (a-z A-Z 0-9 -) C = next pos * = backspace # = save ⚠ Max 32 chars; cursor wraps at end; save with # or long-press C. |
| --- | --- |

# 12. About

The About screen is accessible from the Main Menu without login. It shows firmware and hardware version numbers.

| GH-Ctrl v1.0.0 HW:1.0 FW:1.0 AB-01 About D = back to main menu * = idle |  |
| --- | --- |

# 13. Complete Screen Index

All screens in the LCD GUI listed by ID with access level and parent screen.

| ID | Screen name | Access | Parent / trigger |
| --- | --- | --- | --- |
| S-01 | Idle — Climate & Mode | Public | Idle loop |
| S-02 | Idle — Wind | Public | Idle loop |
| S-03 | Idle — Window States | Public | Idle loop |
| S-04 | Idle — Mode & Date | Public | Idle loop |
| S-05 | Idle — Network Status | Public | Idle loop |
| S-06 | Idle — Session Info | Farmer+ | Idle loop (when logged in) |
| A-01 | Alarm — Wind Override | Public | Overlays idle loop |
| A-02 | Alarm — Wind Sensor Fault | Public | Overlays idle loop |
| A-03 | Alarm — T/RH Sensor Fault | Public | Overlays idle loop |
| A-04 | Alarm — Manual Override (RRK-3) | Public | Overlays idle loop |
| MM-01 | Main Menu | Public | Any key from idle |
| LG-01 | Login — Select Role | Public | MM-01 or auto on restricted access |
| LG-02 | Login — Enter PIN | Public | LG-01 |
| LG-03 | Login — PIN entry in progress | Public | LG-02 |
| LG-04 | Login — Success | Public | LG-02 |
| LG-05 | Login — Failed / attempts remaining | Public | LG-02 |
| LG-06 | Logout confirm | Farmer+ | MM-01 |
| MC-01 | Manual Control — Window list | Farmer+ | MM-01 |
| MC-02 | Manual Control — Window command | Farmer+ | MC-01 |
| MC-03 | Manual Control — Window moving | Farmer+ | MC-02 |
| MC-04 | Manual Control — Command blocked | Farmer+ | MC-02 |
| MC-05 | Manual Control — Dwell wait | Farmer+ | MC-02 |
| ST-01 | Settings Menu | Farmer+ | MM-01 |
| CL-01 | Climate Settings menu | Farmer+ | ST-01 |
| WS-01 | Wind Safety Settings menu | Farmer+ | ST-01 |
| WN-01 | Window Settings menu | Farmer+ | ST-01 |
| TD-01 | Time & Date menu | Farmer+ | ST-01 |
| TD-02 | Set Time | Farmer+ | TD-01 |
| TD-03 | Set Date | Farmer+ | TD-01 |
| TD-04 | Time/Date saved confirmation | Farmer+ | TD-02 / TD-03 |
| NW-01 | Network Settings menu | Admin | ST-01 |
| NW-02 | WiFi Mode toggle | Admin | NW-01 |
| AC-01 | Access Control menu | Admin | ST-01 |
| AC-02 | Enter new PIN | Admin | AC-01 |
| AC-03 | Confirm new PIN | Admin | AC-02 |
| AC-04 | PIN saved confirmation | Admin | AC-03 |
| VE-01 | Value Edit — floating-point numeric | Farmer+ | CL/WS/WN item |
| VE-02 | Value Edit — integer numeric | Farmer+ | WN/TD item |
| VE-03 | Value Edit — option / enum select | Farmer+ | WS item |
| VE-04 | Value Edit — text (SSID) | Admin | NW item |
| AB-01 | About | Public | MM-01 |

End of LCD GUI Design v0.1  —  Greenhouse Ventilation Controller