Implementation Plan

Greenhouse Ventilation Controller

| Document | Implementation Plan |
| --- | --- |
| Project | Greenhouse Ventilation Controller |
| Version | 0.1 (draft) |
| Date | 2026-03-30 |
| Status | Draft |
| Related docs | technicalHardwareDesignSpecification.md, technicalSoftwareDesignSpecification.md, tasks.md, softwareTestPlan.md |

# 1. Introduction

This document defines the ordered implementation plan for the greenhouse ventilation controller. It is split into two independent tracks:

Part A — Hardware: PCB design, fabrication, assembly, and electrical verification.

Part B — Software: Peripheral driver libraries, unit tests, FreeRTOS task development, and integration.

The two tracks can proceed partially in parallel; hardware bring-up requires a working PCB, while software driver libraries and unit tests can be developed and validated on the host before hardware is available.

## Guiding Principles

Libraries first: Every peripheral is encapsulated in a standalone library before any FreeRTOS task is written. This separates hardware interaction from RTOS machinery.

Unit test before integration: Each library is validated with host-side unit tests before it is used inside a FreeRTOS task. Tests run without hardware (mock/stub the hardware layer) and also on target hardware where practical.

RTOS-compatible design: Library APIs must be safe to call from FreeRTOS tasks. No global state that requires external locking, no blocking delays in ISR context, and no dependencies on Arduino::delay() (use vTaskDelay() instead).

Task order by dependency: FreeRTOS tasks are implemented in the order that respects their data and signalling dependencies — foundation tasks before application tasks, application tasks before network tasks.

# 2. Part A — Hardware Implementation

## A1 — Schematic Completion

Goal: A complete, reviewable KiCad schematic with all components, all net connections, and all design decisions resolved.

| Step | Action | Notes |
| --- | --- | --- |
| A1.1 | Resolve Open Issue #7: confirm time source (DS3231 assumed) | Affects RTC schematic symbol and power net |
| A1.2 | Resolve Open Issue #1: confirm RRK-3 alarm relay output signal type | Affects opto-coupler input circuit |
| A1.3 | Complete relay output section (6 relays, opto-isolated 5 V drive, screw terminals) | Verify coil current against 5 V rail budget |
| A1.4 | Complete power supply section (AC-DC HLK-10M24, DC-DC 24 V → 5 V, buffer capacitor) | Size buffer cap for ≥ 1 s sustain at full load |
| A1.5 | Complete RS485 transceiver circuit (SIT65HVD08P, DE/RE control, termination) | Include bus termination resistor (120 Ω) |
| A1.6 | Add ESD and TVS protection on all external-facing connectors | RS485, 24 V sensor supply, relay screw terminals |
| A1.7 | Add decoupling capacitors to all ICs following datasheet recommendations | DS3231, SIT65HVD08P, relay driver |
| A1.8 | Add test points on key signals (UART TX/RX, I2C SDA/SCL, 5 V and 3.3 V rails) | Required for bring-up and production test |
| A1.9 | Perform schematic ERC in KiCad; resolve all errors | No unconnected pins without explicit no-connect markers |
| A1.10 | Peer-review schematic against technicalHardwareDesignSpecification.md | Cross-check all GPIO assignments and voltage levels |

Exit Criteria: ERC passes with zero errors. All open issues resolved. Schematic reviewed and approved.

## A2 — PCB Layout

Goal: A complete, DRC-clean PCB layout that fits within the Multicomp Pro MC001110 enclosure (222 × 146 × 55 mm).

| Step | Action | Notes |
| --- | --- | --- |
| A2.1 | Define board outline matching enclosure mounting pattern | Confirm standoff positions against enclosure datasheet |
| A2.2 | Import net list from completed schematic | Must be based on approved A1 schematic |
| A2.3 | Place connectors first: screw terminals (relay, sensor, power), USB-C, SD card | Connectors define board edge orientations |
| A2.4 | Place LOLIN S3 module centrally; reserve keep-out zone for WiFi antenna | Antenna must not be shielded by copper planes |
| A2.5 | Place relay module(s) in a block, isolated from logic circuitry | Route high-current paths with wide traces |
| A2.6 | Place I2C peripherals (DS3231, LCD header) close to MCU I2C pins | Keep I2C traces short (< 50 mm) |
| A2.7 | Place RS485 transceiver adjacent to UART pins and RS485 screw terminal | Minimise stub length on differential pair |
| A2.8 | Route power planes (24 V, 5 V, GND) before signal traces | Use poured copper fills; check via stitching |
| A2.9 | Route high-current traces (relay coil drive, 24 V input) at ≥ 1 mm width | Calculate trace width from IPC-2221 |
| A2.10 | Route RS485 differential pair as matched length; add 120 Ω termination | Impedance-controlled routing not required at 9600 baud |
| A2.11 | Route I2C and SPI signal traces | Keep away from relay switching traces |
| A2.12 | Add silkscreen labels for all connectors and test points | Include pin numbering and voltage ratings |
| A2.13 | Run DRC; resolve all errors and review all warnings | No unrouted nets; no clearance violations |
| A2.14 | Verify 3D model in KiCad against enclosure dimensions | Check LCD header height against transparent cover |
| A2.15 | Generate fabrication outputs (Gerbers, drill, BOM, pick-and-place) | Follow JLCPCB or equivalent fab house requirements |

Exit Criteria: DRC passes with zero errors. 3D fit verified against enclosure. Fabrication files generated and checked.

## A3 — Fabrication and Assembly

| Step | Action | Notes |
| --- | --- | --- |
| A3.1 | Order PCB (prototype quantity: 5) | Use approved Gerber files from A2 |
| A3.2 | Order all components per BOM | Verify footprints match ordered variants |
| A3.3 | Solder SMD components (DC-DC converter, resistors, capacitors, SIT65HVD08P) | Hot air or reflow; inspect with magnification |
| A3.4 | Solder through-hole components (relay module, screw terminals, DS3231 module) | Mechanical stability required for screw terminals |
| A3.5 | Solder LOLIN S3 module (pin headers or direct SMD pads) | Double-check orientation; module not reversible |
| A3.6 | Install LCD module on standoffs inside enclosure | Connect via ribbon or jumper cable |
| A3.7 | Final assembly into enclosure; dress all cables | Verify cable glands seal correctly for IP67 |

Exit Criteria: Assembled board fits in enclosure. Visual inspection complete. No solder bridges or cold joints.

## A4 — Hardware Bring-Up and Verification

Goal: Verify every hardware subsystem electrically before running firmware.

| Step | Action | Pass Criteria |
| --- | --- | --- |
| A4.1 | Bench power-on: apply 24 VDC (current limited); measure 5 V and 3.3 V rails | 5 V: 4.9–5.1 V; 3.3 V: 3.25–3.35 V; no smoke |
| A4.2 | Measure quiescent current from 24 V rail | Within calculated budget (< 200 mA with no sensors) |
| A4.3 | Flash minimal firmware; verify UART output on serial console | Boot messages appear; no crash |
| A4.4 | Verify I2C bus: scan for DS3231 (0x68) and LCD (0x27) | Both addresses detected |
| A4.5 | Verify DS3231: set time, power-cycle, read back correct time | Time is retained without main power |
| A4.6 | Verify LCD: display test pattern on all 32 character positions | All positions visible through enclosure cover |
| A4.7 | Verify keypad: read all 16 keys individually | Each key generates correct row/column code |
| A4.8 | Verify RS485: connect one Modbus sensor; read a single register | Valid Modbus response received |
| A4.9 | Verify relay outputs: energise each relay individually via firmware | Audible click; continuity measured across contact |
| A4.10 | Verify opto-coupler input: apply 24 V to input; read GPIO | GPIO transitions from low to high (or inverse per schematic) |
| A4.11 | Verify WiFi: connect to AP; ping test | AP visible in scan; HTTP response received |
| A4.12 | Verify SD card (if populated): mount LittleFS; write/read 1 kB file | File contents match after power cycle |
| A4.13 | Verify status LEDs: toggle each LED independently via firmware | All three LEDs illuminate at correct colour |
| A4.14 | Full thermal test at rated load for 30 min | No component exceeds 70 °C case temperature |

Exit Criteria: All A4 checks pass. Results documented in a bring-up report.

# 3. Part B — Software Implementation

Software is developed in two phases:

Phase 1: Standalone peripheral libraries, each validated with unit tests.

Phase 2: FreeRTOS tasks, implemented in dependency order, each validated by integration tests before the next task is built.

## Phase 1 — Peripheral Libraries

Each library encapsulates one hardware peripheral and provides a clean C/C++ API with no FreeRTOS calls in the library code itself (use of vTaskDelay() in drivers is allowed but must be documented). Libraries are covered by unit tests that run on the host using stubs for the Arduino HAL and also on target hardware. A library is accepted when all unit tests pass on both host and target.

| ID | Library | Responsibility | Depends On |
| --- | --- | --- | --- |
| LIB-1 | GPIO Utility | Relay outputs, status LEDs, RS485 DE/RE, opto input | None |
| LIB-2 | I2C Bus | Shared I2C bus wrapper with FreeRTOS mutex (MX1) | None |
| LIB-3 | DS3231 RTC | Real-time clock read/write over I2C | LIB-2 |
| LIB-4 | LCD1602 I2C | PCF8574-based character LCD driver | LIB-2 |
| LIB-5 | Keypad Matrix | 4×4 membrane keypad scan and debounce | None |
| LIB-6 | Modbus RTU | Modbus RTU master over UART + RS485 (FC03, FC04) | LIB-1 |
| LIB-7 | NVS Configuration | ESP32 NVS wrapper for settings and log ring buffer | None |
| LIB-8 | SD Card / LittleFS | File I/O on SD card (SPI) and LittleFS (internal flash) | None |

### LIB-1 — GPIO Utility Library

Abstracts all digital output and input GPIO operations: relay control (active-low), status LEDs, opto-coupler feedback input, and RS485 DE/RE direction control.

API surface:

gpio_init()  — configure all GPIO directions and initial states

gpio_relay_set(channel, state)  — assert/deassert relay (with active-low inversion)

gpio_led_set(led_id, state)  — control LED state

gpio_rs485_direction(tx_enable)  — switch RS485 transceiver direction

gpio_read_opto()  — read opto-coupler input

FreeRTOS compatibility: No RTOS calls; all functions are safe to call from any task context.

Unit tests:

test_relay_default_all_off — verify all relays start deasserted after init

test_relay_set_assert_deassert — toggle one relay; verify GPIO output matches

test_relay_active_low_inversion — verify active-low polarity is applied correctly

test_led_control — verify each LED can be turned on and off independently

test_rs485_direction_tx — verify DE/RE pin state in transmit mode

test_rs485_direction_rx — verify DE/RE pin state in receive mode

test_opto_read_low — stub GPIO returns low; verify function returns inactive

test_opto_read_high — stub GPIO returns high; verify function returns active

### LIB-2 — I2C Bus Library

Wraps the Arduino Wire library for the shared I2C bus (GPIO 21 SDA, GPIO 22 SCL). Provides mutex-aware access so multiple tasks can safely call I2C peripherals.

API surface:

i2c_init(sda_pin, scl_pin, freq_hz)  — initialise bus

i2c_acquire(timeout_ms)  — take the I2C mutex (MX1); returns bool

i2c_release()  — release MX1

i2c_write(addr, data, len)  — write bytes to device

i2c_read(addr, buf, len)  — read bytes from device

i2c_scan()  — return list of responding addresses

FreeRTOS compatibility: i2c_acquire() calls xSemaphoreTake(). Must never be called from ISR context.

Unit tests:

test_i2c_init — verify no error on init with valid pins

test_i2c_acquire_release — acquire and release mutex; verify state

test_i2c_acquire_timeout — simulate mutex held; verify timeout returns false

test_i2c_write_correct_bytes — stub Wire; verify byte sequence sent

test_i2c_read_returns_data — stub Wire; verify read returns stubbed data

test_i2c_scan_finds_address — stub Wire acknowledge; verify address appears in scan result

### LIB-3 — DS3231 RTC Library

Driver for the DS3231 real-time clock over I2C (address 0x68). Provides time read/write. Uses LIB-2 for bus access.

API surface:

rtc_init()  — initialise; verify DS3231 is present

rtc_get_time(datetime_t *dt)  — read current time into struct

rtc_set_time(const datetime_t *dt)  — write time to RTC

rtc_get_temperature()  — read DS3231 internal temperature (float °C)

FreeRTOS compatibility: Calls i2c_acquire(); is blocking; must not be called from ISR.

Unit tests:

test_rtc_init_detects_device — stub I2C returns ACK at 0x68; verify init succeeds

test_rtc_init_absent_device — stub I2C returns NACK; verify init returns error

test_rtc_decode_bcd — verify BCD to binary decode for hours, minutes, seconds

test_rtc_encode_bcd — verify binary to BCD encode for hours, minutes, seconds

test_rtc_get_time_returns_correct_struct — stub I2C read with known BCD bytes; verify datetime fields

test_rtc_set_time_sends_correct_bytes — call set_time; verify I2C write byte sequence

test_rtc_temperature_decode — stub register values; verify float conversion

### LIB-4 — LCD1602 I2C Library

Driver for the Waveshare LCD1602 I2C module (PCF8574 at address 0x27). Provides character and string display. Uses LIB-2 for bus access.

API surface:

lcd_init()  — initialise display (4-bit mode, 2 rows, backlight on)

lcd_clear()  — clear display

lcd_set_cursor(row, col)  — position cursor (row 0–1, col 0–15)

lcd_print(str)  — write null-terminated string at cursor

lcd_print_char(c)  — write single character

lcd_backlight(on)  — control backlight

FreeRTOS compatibility: Calls i2c_acquire(); is blocking; must not be called from ISR.

Unit tests:

test_lcd_init_sequence — verify PCF8574 receives correct 4-bit init nibbles

test_lcd_clear_sends_command — verify clear command byte is sent

test_lcd_set_cursor_row0 — verify DDRAM address for row 0 positions

test_lcd_set_cursor_row1 — verify DDRAM address offset for row 1 (0x40)

test_lcd_print_sends_ascii — write "Hi"; verify PCF8574 receives matching ASCII

test_lcd_backlight_on_off — verify backlight bit in PCF8574 output byte

### LIB-5 — Keypad Matrix Library

Driver for the 4x4 membrane keypad (8 GPIO pins: 4 row outputs, 4 column inputs with pull-ups). Implements hardware matrix scan and software debounce.

API surface:

keypad_init(row_pins[4], col_pins[4])  — configure GPIO directions and pull-ups

keypad_scan()  — scan matrix; returns key code or KEY_NONE (call every 20 ms)

keypad_get_char(keycode)  — map keycode to character (0–9, A–D, *, #)

FreeRTOS compatibility: No RTOS calls; safe to call from any task. Designed for periodic polling from T7.

Unit tests:

test_keypad_no_key_pressed — all columns read high; verify returns KEY_NONE

test_keypad_key_press_row0_col0 — stub col0 low when row0 driven; verify correct key code

test_keypad_key_press_each_key — parameterised test for all 16 keys

test_keypad_debounce_single_transition — key detected after N consecutive identical scans

test_keypad_debounce_rejects_glitch — single-scan low followed by highs does not trigger key event

test_keypad_get_char_mapping — verify all 16 keycodes map to correct characters

### LIB-6 — Modbus RTU Library

Modbus RTU master driver for UART1 (GPIO 17 TX, GPIO 18 RX) via the SIT65HVD08P RS485 transceiver. Implements FC03 (read holding registers) and FC04 (read input registers). Uses LIB-1 for RS485 direction control.

API surface:

modbus_init(uart_port, baud, tx_pin, rx_pin, de_re_pin)  — configure UART and GPIO

modbus_read_registers(slave_id, start_reg, count, buf)  — read count registers; returns error code

modbus_last_error()  — return last error code (timeout, CRC error, exception code)

FreeRTOS compatibility: Uses uart_read_bytes() with timeout; is blocking during Modbus transaction. Must be called from T5 only.

Unit tests:

test_modbus_crc16_known_values — verify CRC16 against known Modbus test vectors

test_modbus_build_request_fc03 — verify request frame bytes for FC03

test_modbus_parse_response_valid — feed valid response frame; verify register values decoded

test_modbus_parse_response_crc_error — corrupt last byte; verify CRC_ERROR returned

test_modbus_parse_response_exception — feed exception frame; verify exception code returned

test_modbus_timeout — stub UART returns no data within timeout; verify TIMEOUT error

test_modbus_register_count_too_large — request > 125 registers; verify rejected

### LIB-7 — NVS Configuration Library

Wrapper around the ESP32 NVS API for persistent configuration and event log ring buffer storage. Provides typed get/set operations for all system settings.

API surface:

nvs_config_init()  — open NVS namespace; load defaults if first boot

nvs_config_get_u16(key, default_val)  — read uint16 setting

nvs_config_set_u16(key, value)  — write uint16 setting

nvs_config_get_float(key, default_val)  — read float setting

nvs_config_set_float(key, value)  — write float setting

nvs_config_get_str / nvs_config_set_str  — read/write string setting

nvs_config_reset()  — erase all settings and restore defaults

nvs_log_append(entry) / nvs_log_read(index, entry)  — ring buffer log I/O

FreeRTOS compatibility: NVS API is thread-safe on ESP32. Callers serialise via MX4.

Unit tests:

test_nvs_default_values_on_first_boot — erase NVS; verify defaults returned

test_nvs_round_trip_u16 — write value; read back; verify equal

test_nvs_round_trip_float — write float; read back; verify within epsilon

test_nvs_round_trip_string — write string; read back; verify equal

test_nvs_reset_restores_defaults — write value; reset; verify default returned

test_nvs_log_append_and_read — append 5 entries; read back; verify order and content

test_nvs_log_ring_wrap — fill beyond capacity; verify oldest entry overwritten

### LIB-8 — SD Card / LittleFS Library

Optional library providing file I/O on the SD card (SPI) and LittleFS (internal flash). Used by T9 (event log) and T11 (web server static files).

API surface:

storage_init(sd_cs_pin)  — mount SD card (SPI); mount LittleFS; report available space

storage_sd_write_append(path, data, len)  — append bytes to file on SD card

storage_sd_read(path, buf, max_len)  — read file from SD card

littlefs_read(path, buf, max_len)  — read file from LittleFS

storage_sd_available()  — returns bool; false if SD card not present

FreeRTOS compatibility: File operations are blocking. Callers serialise via MX5 (LittleFS) or a dedicated SD mutex.

Unit tests:

test_littlefs_read_existing_file — place known file in LittleFS image; verify read content

test_sd_write_append_creates_file — write to new path; verify file exists after flush

test_sd_write_append_grows_file — append twice; verify file length doubles

test_sd_available_false_when_absent — stub SPI returns no card; verify returns false

test_storage_sd_not_blocking_littlefs — SD unavailable; verify LittleFS reads still succeed

## Phase 2 — FreeRTOS Task Implementation

Tasks are implemented in dependency order. A task is not started until all libraries it depends on have passed unit tests and all tasks it depends on have been validated.

| ID | Task | Group | Library Deps | Task Deps |
| --- | --- | --- | --- | --- |
| T1 | Watchdog / Heartbeat | 1 — Foundation | LIB-1 | None |
| T9 | Event Logger | 1 — Foundation | LIB-7, LIB-8 | None |
| T4 | Data Manager | 2 — Data Layer | LIB-7, LIB-3 | T1 |
| T5 | Sensor Poll | 3 — Sensors | LIB-6 | T4 |
| T3 | Safety Monitor | 3 — Safety | LIB-1 | T4, T5 |
| T2 | Relay Controller | 4 — Control | LIB-1 | T1, T9 |
| T6 | Climate Control | 4 — Control | None (uses T4 API) | T2, T3, T4 |
| T7 | Keypad Scan | 5 — UI | LIB-5 | T8 (Q2 consumer) |
| T8 | UI / Display | 5 — UI | LIB-4, LIB-2, LIB-3 | T4, T9 |
| T10 | Network Manager | 6 — Network | ESP-IDF WiFi | T4, T9 |
| T11 | Web Server | 6 — Network | LIB-8 (LittleFS) | T10, T4 |
| T12 | MQTT Client | 6 — Network | None | T10, T4 |
| T13 | OTA Update | 6 — Network | LIB-8 | T11, T10 |

## Group 1 — Foundation Tasks

### T1 — Watchdog / Heartbeat

Group: 1 — Foundation   Library deps: LIB-1 (GPIO Utility)   Task deps: None

Kicks the hardware watchdog every 500 ms and toggles the amber heartbeat LED at 1 Hz (4 Hz during startup). Its liveness confirms the entire system is running. Implemented as a high-priority FreeRTOS software timer or minimal task.

Implementation:

Call esp_task_wdt_reset() every 500 ms.

Call gpio_led_set(LED_HEARTBEAT, toggle) to pulse at 1 Hz in normal mode.

Pulse at 4 Hz until heartbeat_set_normal() is called after startup completes.

Acceptance tests:

test_t1_wdt_kick_period — measure time between WDT kicks; verify within 400–600 ms

test_t1_led_toggle_rate_normal — verify LED toggles at 1 Hz in normal mode

test_t1_led_toggle_rate_startup — verify LED toggles at 4 Hz during startup mode

test_t1_starvation_guard — simulate 600 ms task starvation; verify WDT resets system

### T9 — Event Logger

Group: 1 — Foundation   Library deps: LIB-7 (NVS), LIB-8 (SD / LittleFS)   Task deps: None (Q3 must exist)

Waits on queue Q3 for log_event_t messages from any task. Writes each entry to the NVS ring buffer and optionally to an SD card CSV file. Decouples all log I/O from real-time tasks.

Implementation:

Block on Q3; on message: call nvs_log_append().

If SD card is available, append a CSV line to the log file.

Never block the sender — Q3 must have sufficient depth.

Acceptance tests:

test_t9_receives_log_event — post event to Q3; verify entry appears in NVS log

test_t9_sd_write_when_available — stub SD available; verify CSV line written

test_t9_sd_skip_when_unavailable — stub SD absent; verify NVS write still succeeds

test_t9_queue_depth_not_blocking — post 10 events rapidly; verify none dropped

## Group 2 — Data Layer

### T4 — Data Manager

Group: 2 — Data Layer   Library deps: LIB-7 (NVS), LIB-3 (DS3231)   Task deps: T1 (WDT active)

Central source of truth for all configuration and sensor state. Owns shared data behind MX2, MX3, and MX4. Receives sensor readings from Q6 and configuration updates from Q4. Notifies T3 and T6 of new data.

Implementation:

Read all configuration from NVS at startup; populate in-memory config struct.

Read current time from DS3231 at startup.

Receive sensor readings from Q6; update shared state; send TN1 to T3 and TN2 to T6.

Receive config updates from Q4; validate; persist to NVS.

Expose mutex-protected accessors: data_get_temperature(), data_get_wind_speed(), data_get_config().

Acceptance tests:

test_t4_loads_config_on_startup — stub NVS with known values; verify accessors return them

test_t4_defaults_on_empty_nvs — clean NVS; verify defaults used

test_t4_sensor_reading_update — post reading to Q6; verify accessors return updated values

test_t4_notifies_t3_on_wind_data — post wind reading; verify TN1 sent

test_t4_notifies_t6_on_sensor_data — post T/RH reading; verify TN2 sent

test_t4_config_update_persisted — post config change to Q4; verify NVS updated

test_t4_config_validation_rejects_out_of_range — invalid setpoint; verify rejected

test_t4_mutex_prevents_race — simulate concurrent read during update; verify no data corruption

## Group 3 — Sensor and Safety

### T5 — Sensor Poll

Group: 3 — Sensors   Library deps: LIB-6 (Modbus RTU), LIB-1 (GPIO)   Task deps: T4 (running, Q6 consumer)

Modbus RTU master. Polls SenseCAP S200 (wind speed, wind direction) and FG6485A (temperature, humidity) every 60 s (configurable). Posts readings to Q6 on success; sets fault flags on failure.

Implementation:

Call modbus_read_registers() for SenseCAP S200 wind registers.

Call modbus_read_registers() for FG6485A T/RH registers.

On success: build sensor_reading_t and post to Q6.

On failure: set EG1.SENSOR_FAULT_T or SENSOR_FAULT_W; post fault event to Q3.

Clear fault flag when next successful reading is received.

Acceptance tests:

test_t5_reads_wind_sensor_registers — verify FC03 request sent to SenseCAP S200 address

test_t5_reads_th_sensor_registers — verify FC03 request sent to FG6485A address

test_t5_posts_reading_to_q6 — verify Q6 contains correct decoded reading

test_t5_sets_fault_flag_on_modbus_timeout — stub timeout; verify SENSOR_FAULT_W set

test_t5_clears_fault_flag_on_recovery — fault set; next poll succeeds; verify cleared

test_t5_respects_poll_interval — verify T5 does not poll faster than configured interval

### T3 — Safety Monitor

Group: 3 — Safety   Library deps: LIB-1 (GPIO)   Task deps: T4 (running), T5 (producing data)

Blocks on TN1 from T4. On each notification reads wind speed and direction via T4 accessors. Issues CLOSE_ALL to Q1 when wind speed exceeds v_max or wind direction is in the exclusion zone. Always active regardless of operating mode.

Implementation:

Block on TN1.

Read wind speed and direction from T4 (mutex-protected).

If speed >= v_max OR direction in exclusion zone: post CLOSE_ALL to Q1; set EG1.WIND_OVERRIDE.

Clear WIND_OVERRIDE after wind returns to safe values for hold-off period.

Post all safety events to Q3.

Acceptance tests:

test_t3_no_action_below_threshold — wind = v_max - 1; verify no CLOSE_ALL command

test_t3_triggers_on_speed_threshold — wind = v_max; verify CLOSE_ALL posted

test_t3_triggers_on_direction_exclusion — direction in exclusion zone; verify CLOSE_ALL

test_t3_wind_override_flag_set — trigger safety; verify EG1.WIND_OVERRIDE set

test_t3_hold_off_prevents_premature_clear — wind drops; verify flag not cleared until hold-off expires

test_t3_active_in_standby_mode — system in Standby; verify T3 still triggers

test_t3_logs_safety_event — verify event posted to Q3

## Group 4 — Control and Relay

### T2 — Relay Controller

Group: 4 — Control   Library deps: LIB-1 (GPIO)   Task deps: T1 (running), T9 (running)

Sole owner of all 6 relay GPIO outputs. Receives actuation_cmd_t messages from Q1. Runs per-channel window state machines, enforces mutual exclusion, and manages dwell timers. Motor travel times: M1 = 21 s, M2 = 21 s, M3 = 171 s.

Implementation:

Block on Q1.

Enforce OPEN/CLOSE mutual exclusion per channel.

Run state machine: CLOSED -> MOVING -> OPEN and reverse.

Apply dwell timers; defer conflicting commands until dwell expires.

CLOSE_ALL (from T3) preempts dwell timers immediately.

Monitor opto-coupler input; on transition set EG1.MANUAL_OVERRIDE; send TN3 to T6.

Post log event to Q3 for every relay transition.

Acceptance tests:

test_t2_default_state_all_closed — startup; verify all relay GPIO deasserted

test_t2_open_command_asserts_open_relay — post OPEN M1; verify OPEN relay GPIO asserted

test_t2_mutual_exclusion — verify OPEN and CLOSE never simultaneously asserted

test_t2_travel_time_m1 — post OPEN M1; verify relay de-asserts after 21 s

test_t2_travel_time_m3 — post OPEN M3; verify relay de-asserts after 171 s

test_t2_dwell_timer_blocks_rapid_reversal — OPEN then immediate CLOSE; verify CLOSE deferred

test_t2_close_all_preempts_dwell — WIND_OVERRIDE CLOSE_ALL; verify windows closed immediately

test_t2_opto_input_triggers_manual_override — assert opto; verify MANUAL_OVERRIDE set and TN3 sent

test_t2_logs_every_command — verify Q3 receives log event for each relay transition

### T6 — Climate Control

Group: 4 — Control   Library deps: None (uses T4 API)   Task deps: T2, T3, T4 (all running)

Blocks on TN2 from T4. Evaluates T/RH against setpoints with hysteresis. Runs conflict resolution algorithm when temperature and humidity demands conflict. Posts OPEN/CLOSE commands to Q1. Inhibited during WIND_OVERRIDE, MANUAL_OVERRIDE, or Standby mode.

Implementation:

Block on TN2.

Read T, RH, and setpoints from T4 accessors.

Evaluate temperature demand (open/close); evaluate humidity demand.

Apply conflict resolution if demands conflict.

Post commands to Q1.

Check EG1 flags before posting; skip if any override is active.

On TN3 (manual override): inhibit until EG1.MANUAL_OVERRIDE cleared.

Acceptance tests:

test_t6_no_action_within_setpoint_band — T and RH within deadband; verify no command posted

test_t6_opens_window_on_high_temperature — T > setpoint + hysteresis; verify OPEN command

test_t6_closes_window_on_low_temperature — T < setpoint - hysteresis; verify CLOSE command

test_t6_opens_window_on_high_humidity — RH > RH_max; verify OPEN command

test_t6_conflict_resolution — T high and RH low; verify priority per FR-CC06

test_t6_inhibited_during_wind_override — WIND_OVERRIDE set; verify no commands

test_t6_inhibited_during_manual_override — MANUAL_OVERRIDE set; verify no commands

test_t6_inhibited_in_standby — Standby mode; verify no commands

test_t6_resumes_after_override_cleared — override cleared; verify commands resume

## Group 5 — User Interface

### T7 — Keypad Scan

Group: 5 — UI   Library deps: LIB-5 (Keypad Matrix)   Task deps: T8 (must be running to consume Q2)

Periodic 20 ms software timer or dedicated task. Calls keypad_scan() each tick. Posts key_event_t to Q2 on debounced key press. No key repeat.

Implementation:

Call keypad_scan() every 20 ms.

On debounced key press: post key_event_t to Q2.

No repeat event for held keys.

Acceptance tests:

test_t7_no_event_when_no_key — no key pressed; verify Q2 empty

test_t7_posts_event_on_key_press — stub key press; verify Q2 receives event

test_t7_debounce_suppresses_glitch — single scan low; verify no event

test_t7_no_repeat_on_held_key — key held 200 ms; verify exactly one event

test_t7_scan_rate_20ms — measure callback period; verify within 18–22 ms

### T8 — UI / Display

Group: 5 — UI   Library deps: LIB-4 (LCD1602), LIB-2 (I2C), LIB-3 (DS3231)   Task deps: T4, T9 (running)

Refreshes LCD every 200 ms with T, RH, wind speed, window states, mode, and alarms. Processes key events from Q2 through a menu FSM. Manages PIN sessions. Posts config changes to Q4 and manual commands to Q1.

Implementation:

Wake every 200 ms; read data from T4 accessors; render status screen via LIB-4.

Process Q2 key events through menu FSM.

PIN entry: hash input against stored bcrypt hash (or equivalent); elevate session on match.

Post config changes to Q4.

Receive network status from Q5; display connection indicator.

Post UI events to Q3.

Acceptance tests:

test_t8_home_screen_renders_temperature — stub T4; verify LCD receives matching string

test_t8_home_screen_renders_window_states — stub T4 states; verify LCD output

test_t8_menu_navigation_down — post DOWN key; verify state transition

test_t8_menu_navigation_enter — post ENTER in menu; verify submenu entered

test_t8_pin_entry_accepted — correct PIN; verify session elevated

test_t8_pin_entry_rejected — wrong PIN; verify session unchanged

test_t8_config_change_posted_to_q4 — change setpoint; verify Q4 receives update

test_t8_alarm_shown_on_wind_override — EG1.WIND_OVERRIDE set; verify alarm on LCD

test_t8_network_status_shown — CONNECTED event on Q5; verify displayed

## Group 6 — Network Tasks

Network tasks run on Core 0. They depend on T4 (Data Manager) but are independent of Groups 4 and 5. They can be developed after T4 is validated.

### T10 — Network Manager

Group: 6 — Network   Library deps: ESP-IDF WiFi API   Task deps: T4 (config), T9 (logging)

Manages WiFi AP and client lifecycle. Triggers NTP synchronisation on client connect and updates DS3231. Posts network status events to Q5.

Implementation:

Start in AP mode if no client credentials in NVS.

Connect as client if credentials stored.

On connect: trigger NTP; update DS3231 time; update T4.

Post net_status_event_t to Q5.

Reconnect automatically on disconnect.

Acceptance tests:

test_t10_starts_ap_mode_without_credentials — empty NVS; verify AP mode

test_t10_connects_client_with_credentials — credentials in NVS; verify connect attempt

test_t10_ntp_sync_on_connect — stub NTP; verify DS3231 time set

test_t10_posts_status_on_connect — verify Q5 receives CONNECTED event

test_t10_posts_status_on_disconnect — simulate disconnect; verify DISCONNECTED event

test_t10_reconnects_after_timeout — simulate disconnect then available; verify reconnect

### T11 — Web Server

Group: 6 — Network   Library deps: LIB-8 (LittleFS)   Task deps: T10 (WiFi), T4 (data)

ESPAsyncWebServer serving HTML/CSS/JS from LittleFS. Three-state session model: Normal / Farmer / Admin. Endpoints for sensor data, window status, configuration, manual commands, and log download.

Implementation:

Serve static files from LittleFS.

Implement session cookie authentication with bcrypt PIN hashes.

POST /config: validate; post to Q4.

POST /window: require Farmer role; post command to Q1.

Acceptance tests:

test_t11_serves_index_html — GET /; verify 200 with HTML

test_t11_serves_sensor_data_json — GET /api/sensors; verify JSON fields

test_t11_requires_auth_for_config — unauthenticated POST /config; verify 401

test_t11_config_post_accepted — authenticated POST; verify Q4 updated

test_t11_manual_command_requires_farmer — Normal session OPEN; verify rejected

test_t11_manual_command_posts_to_q1 — Farmer session OPEN M1; verify Q1 updated

### T12 — MQTT Client

Group: 6 — Network   Library deps: None   Task deps: T10 (WiFi), T4 (data)

Optional feature; enabled via NVS. Publishes sensor data and window states to configured broker. Subscribes to command topics and posts validated commands to Q1.

Implementation:

Check NVS for MQTT enabled flag; skip connection if disabled.

Publish T, RH, wind, window states on configurable interval.

Subscribe to command topics; validate and post to Q1.

Acceptance tests:

test_t12_disabled_when_not_configured — MQTT disabled; verify no connection

test_t12_publishes_sensor_data — stub broker; verify correct topic and payload

test_t12_subscribes_to_command_topic — verify subscribe called on connect

test_t12_command_received_posts_to_q1 — valid command on topic; verify Q1 updated

test_t12_ignores_invalid_payload — malformed payload; verify Q1 not updated

### T13 — OTA Update

Group: 6 — Network   Library deps: LIB-8 (LittleFS)   Task deps: T11 (running), T10 (WiFi)

Triggered by Administrator session in web UI. Dual-bank OTA via ESP-IDF OTA API. Sets EG1.OTA_IN_PROGRESS during update. Reverts to previous bank after 3 consecutive failures.

Implementation:

Require Administrator session from T11.

Set EG1.OTA_IN_PROGRESS.

Flash new firmware to inactive bank.

On success: clear flag; reboot.

On failure: increment counter; rollback after 3 failures.

Acceptance tests:

test_t13_requires_admin_role — Farmer session triggers OTA; verify rejected

test_t13_sets_ota_in_progress_flag — start OTA; verify flag set

test_t13_clears_flag_on_completion — OTA completes; verify flag cleared

test_t13_rollback_after_three_failures — stub OTA to fail; verify rollback after 3rd

## Phase 3 — Integration and System Test

After all tasks are individually validated, the following integration tests verify end-to-end behaviour. These tests align with the acceptance criteria in softwareTestPlan.md.

| Test ID | Scenario | Validation |
| --- | --- | --- |
| INT-01 | Startup sequence | All 3 windows close within 215 s; HB LED pulses at 4 Hz then drops to 1 Hz |
| INT-02 | Temperature-driven window opening | T rises above setpoint; T6 posts OPEN; T2 runs relay for correct travel time |
| INT-03 | Wind override | Wind speed breaches threshold mid-opening; T3 posts CLOSE_ALL; windows close; climate control inhibited |
| INT-04 | Wind override clears | Wind drops; hold-off expires; climate control resumes |
| INT-05 | Manual override via RRK-3 | RRK-3 relay de-energises; opto input changes; T2 detects override; T6 inhibited; UI shows alarm |
| INT-06 | Keypad menu — setpoint change | Operator navigates menu; changes T setpoint; T4 persists to NVS; T6 uses new value |
| INT-07 | Web UI — manual window command | Administrator opens M1 via web UI; T11 posts to Q1; T2 opens window |
| INT-08 | Power interruption | Disconnect 24 V for 5 s; restore; verify DS3231 retains time; NVS retains config; system resumes |
| INT-09 | MQTT publish | Broker connected; verify T, RH, wind, window state published every interval |
| INT-10 | OTA update | Upload firmware image via web UI; verify successful flash and boot from new partition |
| INT-11 | Conflict resolution | T high and RH low simultaneously; verify window behaviour follows priority in FR-CC06 |
| INT-12 | WDT recovery | Block T1 artificially for > WDT timeout; verify system resets and restarts cleanly |

# 4. Dependency Overview

Libraries (Phase 1)
  LIB-1  GPIO Utility
  LIB-2  I2C Bus
    LIB-3  DS3231 RTC         (needs LIB-2)
    LIB-4  LCD1602 I2C        (needs LIB-2)
  LIB-5  Keypad Matrix
  LIB-6  Modbus RTU           (needs LIB-1)
  LIB-7  NVS Configuration
  LIB-8  SD Card / LittleFS

FreeRTOS Tasks (Phase 2)
  Group 1 — Foundation
    T1  Watchdog / Heartbeat  (LIB-1)
    T9  Event Logger          (LIB-7, LIB-8)

  Group 2 — Data Layer
    T4  Data Manager          (LIB-7, LIB-3)  | needs T1

  Group 3 — Sensors & Safety
    T5  Sensor Poll           (LIB-6)          | needs T4
    T3  Safety Monitor        (LIB-1)          | needs T4, T5

  Group 4 — Control & Relay
    T2  Relay Controller      (LIB-1)          | needs T1, T9
    T6  Climate Control                        | needs T2, T3, T4

  Group 5 — UI
    T7  Keypad Scan           (LIB-5)          | needs T8 (Q2 consumer)
    T8  UI / Display          (LIB-4, LIB-2, LIB-3)  | needs T4, T9

  Group 6 — Network (Core 0)
    T10  Network Manager                       | needs T4, T9
    T11  Web Server           (LIB-8)          | needs T10, T4
    T12  MQTT Client                           | needs T10, T4
    T13  OTA Update           (LIB-8)          | needs T11, T10

# 5. Open Issues That Block Implementation

The following open issues from the design documents must be resolved before the affected implementation steps begin.

| Issue | Blocks | Decision Required |
| --- | --- | --- |
| Open Issue #1 RRK-3 alarm relay output signal type | A1 schematic, LIB-1 unit tests, T2 acceptance tests | Confirm signal polarity (NO/NC) and voltage level of RRK-3 alarm output |
| Open Issue #7 Time source (DS3231 vs GNSS vs DCF77) | A1 schematic, LIB-3 development, T4 time initialisation | Confirm DS3231 as primary time source (current design assumption) |

Both issues are expected to be resolved in favour of the current design assumptions (DS3231; opto-coupled feedback input). If a different time source is selected, LIB-3 and the T4 initialisation sequence must be revised accordingly.

End of Implementation Plan v0.1